Design & Access Statement

Statements to Accompany Applications for Listed Building Consent

Project Details

- Site Address: Town Hall, Town Hall Approach Road, London, N15 4RX

- Applicant: Mr Peter Griffin

- Date: 09 February 2026

- Proposal: Installation of a telecommunications cable within a Grade II listed building to provide fibre connectivity

Brief Description of Development

The proposed works involve the installation of 60 metres of fibre telecommunications cable from an existing fibre box in the basement to a communications cabinet in the ground floor office.

Explain the design principles and concepts that have been applied to the works:

Minimal intervention – routing the cable through service areas and existing containment wherever possible.

Reversibility – all fixings are removable and can be reinstated without harm.

Avoidance of historic fabric – new drilling is limited to non-original surfaces and modern service routes.

Visual discretion – cable runs are concealed or placed out of sight within utilitarian areas.

Protection of significance – ensuring no impact to the Town Hall’s important architectural or decorative spaces.

Explain how the design principles and concepts that have been applied to the works take account of the special architectural or historic importance of the building;

The works will proceed as follows:

1.	The cable will originate from the existing fibre box in the basement.

2.	It will run neatly along the basement tunnel wall, aligned with existing cabling.

3.	The route will then rise into the ground floor staircase area through an existing hole, avoiding the need for any new intervention at this point.

4.	A single drilled hole will be required to enable the cable to enter existing trunking that leads to the ground floor communications room.

5.	The cable will then run through existing trunking and containment for a considerable distance, following the route of other modern services.

6.	It will terminate in the customer communications cabinet located within the ground floor office communications room.

For the majority of the run, the cable will be concealed within existing containment or positioned unobtrusively alongside current infrastructure. All fixings will be lightweight and fully reversible

Explain how the design principles and concepts that have been applied to the works take account of the building’s setting

The proposed works are minor when taking in to account the scale and proportionate to the building’s size and architectural character. This prevents the introduction of elements that would dominate or detract from the historic setting.

List of further information submitted as part of this Statement:

Heritage Statement