| PROTECTED TREE REPORT (TREE WORKS) |  |
| --- | --- |
| Please Scroll to Page 2 | Haringey Council, L4, Alexandra House, Station Road, N22 7TY |

| # Application number | HGY/2026/1980 |
| --- | --- |
| # Application type | Tree Preservation Order application |
| # Site Address: | Blanche Nevile School, Burlington Road, Hornsey, London, N10 1NJ |
| # Proposal | Tree Number - T1 Tree Type - Oak (Quercus robur) Approx. Diameter at 1.5m - 0.64 Approx. Height - 14.5m Approx. Crown Spread Diameter - 10m Location - See map Service - Crown Reduction Work Required - Reduce height by up to 2.5m and lateral spread on all sides by up to 2m, back to previous pruning points, to finished crown dimensions of 12m height and 6m lateral spread Reason - As part of continued maintenance to manage the size of the tree Priority Code - B Tree Number - T2 Tree Type - Oak (Quercus robur) Approx. Diameter at 1.5m - 0.67m Approx. Height - 15.5m Approx. Crown Spread Diameter - 12m Location - See map Service - Crown Reduction Work Required - Reduce height by up to 2.5m and lateral spread on all sides by up to 2m, back to previous pruning points, to finished crown dimensions of 13m height and 8m lateral spread Reason - As part of continued maintenance to manage the size of the tree Priority Code – B |

| # RECOMMENDATION: | # No Objection |  |
| --- | --- | --- |
| # Application date | 14/07/2026 |  |
| # Tree Inspector | DM | Date: 7th October 2026 |

Site

The mature Oak trees T1 & T2 as seen in the photographs, and marked on the plan, are situated in the grounds of the property. They are publicly visible.

A site visit was deemed necessary to determine the appropriateness of the proposed works 21/08/2026 at approximately 11:57am).

The trees are protected by a Tree Preservation Order (TPO)

Proposal

Tree works proposed.

Tree Number - T1

Tree Type - Oak (Quercus robur)

Approx. Diameter at 1.5m - 0.64

Approx. Height - 14.5m

Approx. Crown Spread Diameter - 10m

Location - See map

Service - Crown Reduction

Work Required - Reduce height by up to 2.5m and lateral spread on all sides by up to 2m, back to previous pruning points, to finished crown dimensions of 12m height and 6m lateral spread

Reason - As part of continued maintenance to manage the size of the tree

Priority Code - B

Tree Number - T2

Tree Type - Oak (Quercus robur)

Approx. Diameter at 1.5m - 0.67m

Approx. Height - 15.5m

Approx. Crown Spread Diameter - 12m

Location - See map

Service - Crown Reduction

Work Required - Reduce height by up to 2.5m and lateral spread on all sides by up to 2m, back to previous pruning points, to finished crown dimensions of 13m height and 8m lateral spread

Reason - As part of continued maintenance to manage the size of the tree

Priority Code - B

Issues

I hold no objections to the reasonable proposed pruning works.

The specification is within standard good arboricultural practice and husbandry.

Recommendation

No Objection