Development not permitted

M.1  Development is not permitted by Class M—

| (a)if the cumulative footprint of any erection, extension or alteration under Class M on or after the relevant date would exceed the greater of— | Complies. Proposed footprint is 104.4 m² (building 9.8 m x 9.0 m = 88.2 m², plus 1.5 m covered veranda and 0.3 m roof overhang), or 111.0 m² including the access ramp (conservative figure). The applicable limit is the greater of (i) and (ii) below, approx. 411 m². The relevant date for schools is 21 April 2021. School to confirm that no other Class M development has been built since that date. |
| --- | --- |
| (i)25% of the cumulative footprint of the school, [college,] university, prison or hospital buildings as it was on [the relevant date]; or | Existing school buildings footprint approx. 1,645 m²: EFAA 1,100 m² (55 m x 20 m), EFAB approx. 390 m², EFAC approx. 155 m² (see drawing EPS-SP-01). 25% = approx. 411 m². The proposal (111.0 m²) is approx. 6.7% of the existing footprint. |
| (ii)250 square metres;] | 250 m². The greater limit is therefore approx. 411 m² under (i); the proposal (111.0 m²) is well within either limit. |
| (ba)in the case of a school, where any land adjacent to the site is used for a purpose within Part C of the Schedule to the Use Classes Order (residential purposes), if any part of the proposed development is within 5 metres of the boundary of the curtilage of that residential land;] | Complies. The nearest part of the proposal (access ramp) is 9.5 m from the west site boundary; the building wall is 11.0 m from the west boundary and 17.0 m from the north boundary. No part of the development is within 5 m of any adjacent residential curtilage (see drawing EPS-SP-02). |
| (c)if, as a result of the development, any land used as a playing field at any time in the 5 years before the development commenced and remaining in this use could no longer be so used; | Complies. The building is sited on the existing tarmac hard play area north of the MUGA, outside the school playing field boundary, with a minimum 3.1 m clearance to the MUGA fence. No playing field land is lost. |
| (d)if the height of any new building erected would [exceed—] | Complies – see (i) below. |
| [ (i)if within 10 metres of a boundary of the curtilage of the premises, 5 metres; or | This limit applies, as the ramp is 9.5 m from the west boundary. Height of the new building is approx. 3.0 m (2,987 mm to top of roof fascia per Cabins for Schools drawings Rev A, plus ground-screw base clearance – installer to confirm), below the 5 m limit. |
| (ii)in all other cases, 6 metres;] | Not applicable (5 m limit applies). The proposal would also comply with 6 m. |
| (e)if the height of the building as extended or altered would exceed— | Not applicable – the proposal is a new detached building, not an extension or alteration of an existing building. |
| (i)if within 10 metres of a boundary of the curtilage of the premises, the lesser of the height of the building being extended or altered or] 5 metres; or | Not applicable. |
| (ii)in all other cases, the height of the building being extended or altered; | Not applicable. |
| [ (ea)if the height of any rooftop structure would exceed 1.5 metres above the height of the new building or the building as extended or altered (as the case may be);] | Complies – no rooftop structures are proposed (single-storey building with EPDM membrane roof). |
| (f)if the development would be within the curtilage of a listed building; or | To be confirmed by the school/council. We are not aware of any listed building within the school site. |
| (g)unless— | Complies – see (i) below. |
| (i)in the case of school, college or university buildings, the predominant use of the existing buildings on the premises is for the provision of education; | Complies. The predominant use of the existing buildings is education (Earlham Primary School, community primary school, ages 4–11). The new building will be an Early Years classroom. |
| (ii)in the case of hospital buildings, the predominant use of the existing buildings on the premises is for the provision of any medical or health services | Not applicable. |
| (iii)in the case of prison buildings— | Not applicable. |
| (aa)the predominant use of the existing buildings on the premises is for the confinement of prisoners; and] | Not applicable. |
| (bb)in the case of a closed prison, the development does not involve the erection, extension or alteration of any building beyond the perimeter as it stood on the relevant date.] | Not applicable. |

Conditions

| M.2 Development is permitted by Class M subject to the following conditions— | Response |
| --- | --- |
| (a)the development is within the curtilage of an existing school, college, university [F17, prison] or hospital; | Complies. The proposal is within the school site boundary (red line on EPS-SP-01), in the north-west corner of the site, at least 9.5 m from any boundary. |
| (b)the development is only used as part of, or for a purpose incidental to, the use of that school, college, university [prison] or hospital; | Complies. The building will be used solely as an Early Years classroom with WCs, as part of the use of Earlham Primary School. |
| (e)where proposed development under Class M relates to the erection, extension or alteration of a school building that results in an increase in the school’s published admission number, the developer must, within a period of six months starting with the date the development is completed, submit to the local planning authority a travel plan for the site; | School to confirm whether the new classroom increases the published admission number. If it does, a travel plan must be submitted to Haringey within six months of completion. If it re-houses existing provision with no increase, this condition does not apply. |