4 The Gables N10 3EA

Date of survey 30/05/2023

Rear bedroom windows

The existing windows are softwood and glazed with 3mm clear float

They are original to the property

There has been an historical leak of the guttering to the left hand side of the double window

which has now been rectified but has caused water ingress to damage the internal plaster wall and sections of the window itself

although these have now been filled and painted. The sills have been cut back along the front at some point which means that they are not bridging

the concrete sills. There is a mastic seal along the front edge to try and weather this but it's already lifting so water can now penetrate between

the concrete sill and the underside of the wooden sill. The windows also have cables running through the sills which again can cause water ingress over time.

The pulley stiles have also collapsed at pocket level and the pointing is coming away between the vertical reveals and the windows themselves.

The windows are loose fitting, rattling and drafty and in my view have now reached the point of being beyond repair

Brgds

Graham Sandford

GMS Joinery

020 8883 7462

www.gmsjoinery.co.uk