|  |  | Our ref: HGY/2023/2642 Contact: Cameron Sturges Date: 30/11/2023 |
| --- | --- | --- |
| Town and Country Act 1990 (As amended) |  |  |
| Location: | Alexandra Park Secondary School, Bidwell Gardens, Wood Green, London, N11 2AZ |  |
| Proposal: | Erection of three-storey side extension to provide additional sixth form accommodation (Use Class F1) including access, landscaping, biodiversity enhancements, drainage, cycle parking, renewable energy and sustainability measures. |  |

Carbon Management Response 12/12/2023

In preparing this consultation response, we have reviewed:

- Energy Statement prepared by CBG Consultants (dated 12 Dec 2023)

- Sustainability Statement prepared by Thinking Buildings (dated 25 Sep 2023)

- Relevant supporting documents.

- Summary

The development achieves a reduction of 37% carbon dioxide emissions on site, which is supported in principle. Further information needs to be provided in relation to the Energy Strategy and Sustainability Strategy. This should be addressed prior to the determination of the application or should be addressed through planning conditions.

Planning conditions have been recommended to secure the benefits of the scheme.

- Energy – Overall

Policy SP4 of the Local Plan Strategic Policies, requires all new development to be zero carbon (i.e. a 100% improvement beyond Part L (2021)). The London Plan (2021) further confirms this in Policy SI2.

The overall predicted reduction in CO2 emissions for the development shows an improvement of approximately 37% in carbon emissions with SAP10.2 carbon factors, from the Baseline development model (which is Part L 2013 compliant). This represents an annual saving of approximately 1.2 tonnes of CO2 from a baseline of 3.4tCO2/year.

| Non-residential (SAP10.2 emission factors) |  |  |  |
| --- | --- | --- | --- |
|  | Total regulated emissions (Tonnes CO2 / year) | CO2 savings (Tonnes CO2 / year) | Percentage savings (%) |
| Part L 2021 baseline | 3.4 |  |  |
| Be Lean | 3.2 | 0.2 | 7% |
| Be Clean | 3.2 | 0.0 | 0% |
| Be Green | 2.2 | 1.0 | 30% |
| Cumulative savings |  | 1.2 | 37% |
| Carbon shortfall to offset (tCO2) | 2.2 |  |  |
| Carbon offset contribution | £95 x 30 years x 2.2 tCO2/year = £6,270 |  |  |

Action:

- Please submit the GLA’s Carbon Emission Reporting Spreadsheet.

Energy Use Intensity / Space Heating Demand

Applications are required to report on the total Energy Use Intensity and Space Heating Demand, in line with the GLA Energy Assessment Guidance (June 2022). The Energy Strategy should follow the reporting template set out in Table 5 of the guidance, including what methodology has been used. EUI is a measure of the total energy consumed annually, but should exclude on-site renewable energy generation and energy use from electric vehicle charging.

| Building type | EUI (kWh/m2/year) | Space Heating Demand (kWh/m2/year) | Methodology used |
| --- | --- | --- | --- |
| School | 25.4 | 4.7 | TBC |

Energy – Lean

The applicant has proposed a saving of 0.2 tCO2 in carbon emissions (7%) through improved energy efficiency standards in key elements of the build, based on SAP10.2 carbon factors. However, it is recommended to further improve the energy efficiency following the fabric first approach of the energy hierarchy.

The following u-values, g-values and air tightness are proposed:

| Floor u-value | 0.13 W/m2K |
| --- | --- |
| External wall u-value | 0.16 W/m2K |
| Roof u-value | 0.13 W/m2K |
| Door u-value | 1.40 W/m2K |
| Window u-value | 1.40 W/m2K |
| G-value | 0.40 |
| Air permeability rate | 3 m3/hm2 @ 50Pa |
| Ventilation strategy | Mechanical ventilation with heat recovery (80% efficiency) |
| Thermal bridging | Accredited Construction Details |
| Low energy lighting | 100% |
| Heating system Be Lean only | ASHP with SCOP 3.0 |

London Plan Policy SI4 requires developments to minimise adverse impacts on the urban heat island, reduce the potential for overheating and reduce reliance on air conditioning systems. Through careful design, layout, orientation, materials and incorporation of green infrastructure, designs must reduce overheating in line with the Cooling Hierarchy.

Actions:

- The u-values input in the BRUKL sheets for Be Lean scenario does not correspond with the values reported in the energy strategy. Please amend these values in the revised energy statement.

- Please identify on a plan where the MVHR units will be located within the dwellings. The units should be less than 2m away from external walls. This detail can also be conditioned.

- Demonstrate the cooling hierarchy has been followed, and specify which overheating mitigation measures are proposed to reduce the overheating risk within the proposed design:

  - Internal heat generation, i.e. heat distribution infrastructure

  - Heat entering building, i.e. shutters, trees, vegetation, blinds

  - Manage heat through thermal mass and high ceilings

  - Passive ventilation, i.e. openable windows, shallow floorplates, dual aspect, stack effect

  - Mechanical ventilation, i.e. free cooling from outside air in shade, by-pass for summer mode

Energy – Clean

The applicant is not proposing any Be Clean measures. The site is not within reasonable distance of a proposed Decentralised Energy Network (DEN). A Combined Heat and Power (CHP) plant would not be appropriate for size of development.

Energy – Green

As part of the Be Green carbon reductions, all new developments must achieve a minimum reduction of 20% from on-site renewable energy generation to comply with Policy SP4.

The application has reviewed the installation of various renewable technologies. The report concludes that air source heat pumps (ASHPs) and solar photovoltaic (PV) panels are the most viable options to deliver the Be Green requirement. A total of 1.2 tCO2 (30%) reduction of emissions are proposed under Be Green measures.

The array of 32 panels would be mounted on the roof, at a 10° angle. Other roof space have not been considered due to the shading from neighbouring buildings.

Two air-to-water ASHP system(s) (min. SCOP of 3.0) will provide hot water and heating to the development utilizing a pre heat cylinder.

Actions:

- A living roof should be installed under the solar PV, or if this is not feasible, the roof should be light coloured to reduce solar heat gains and to improve efficiency of the solar panels.

- Please identify on the plans where the internal and external parts of the air source heat pumps will be located and how the units will be mitigated in terms of visual and noise impact.

- How much of the heating/hot water demand will be met by the proposed type of heat pump? If this cannot be met fully, how will this be supplemented?

- Carbon Offset Contribution

A carbon shortfall of 2.2 tCO2/year remains. The remaining carbon emissions will need to be offset at £95/tCO2 over 30 years.

- Overall Sustainability

Policy DM21 of the Development Management Document requires developments to demonstrate sustainable design, layout and construction techniques. The Sustainability section in the report sets out the proposed measures to improve the sustainability of the scheme, including transport, health and wellbeing, materials and waste, water consumption, flood risk and drainage, biodiversity, climate resilience, energy and CO2 emissions and landscape design.

The development achieves an Urban Greening Factor of 0.11, which does not comply with the interim minimum target of 0.3 for predominantly non-residential developments. The bio-diversity net gain assessment exceeds the 10% biodiversity net gain as set out in the accompanying reports.

Sustainability - Non-Domestic BREEAM Requirement

Policy SP4 requires all new non-residential developments to achieve a BREEAM rating ‘Very Good’ (or equivalent), although developments should aim to achieve ‘Excellent’ where achievable.

The applicant has prepared a BREEAM Pre-Assessment Report for the development. Based on this report, a score of 56.23 % is expected to be achieved, equivalent to ‘Very Good’ rating.

Targeting such a low score will risk not achieving ‘Very Good’ as a very minimum, and does not demonstrate the ambition to deliver a more sustainable development.

Actions:

- A table should be submitted to demonstrate which credits will be met, how many are met out of the total available, under which category, which could be achieved and which will not be met. This needs to include justification where targets are not met or ‘potential’ credits (where they are available under the Shell and Core assessment). This will enable better assessment of which credits.

Sustainability – Urban Greening and Biodiversity

All development sites must incorporate urban greening within their fundamental design, in line with London Plan Policy G5. London Plan Policy G6 and Local Plan Policy DM21 require proposals to manage impacts on biodiversity and aim to secure a biodiversity net gain. Additional greening should be provided through high-quality, durable measures that contribute to London’s biodiversity and mitigate the urban heat island impact. This should include tree planting, shrubs, hedges, living roofs, and urban food growing. Specifically, living roofs and walls are encouraged in the London Plan. Amongst other benefits, these will increase biodiversity and reduce surface water runoff.

The development achieves an Urban Greening Factor of 0.11, which does not comply with the interim minimum target of 0.3 for predominantly non-residential developments in London Plan Policy G5. It is recommended to explore further measures to enhance the urban greening factor such as living roofs.

The Biodiversity Net Gain assessment exceeds the 10% requirement as set out in the Environment Act 2021.

Actions:

- Please explore measures to improve the Urban Greening Factor including but not limited to living roofs, trees, etc.

- Conclusion

Overall, it is considered that the application can be supported from a carbon management and sustainability point of view.

Planning Obligations Heads of Terms

- Payment of carbon offset contribution prior to the above ground construction works at £6,270, based on £2,850 per tonne of carbon emissions.

Planning Conditions

Condition have been drafted including the action points as mentioned above. Conditions to be secured.

Energy Strategy

(a) Prior to the commencement of development, a revised Energy Statement shall be submitted and approved by the Local Planning Authority. This shall be based on the submitted Energy Statement prepared by CBG Consultants (dated 12 Dec 2023), delivering a minimum site-wide carbon emission reduction of 37% (SAP 10.2 emission factors) from a Building Regulations 2021 Part L compliant building. The revised strategy shall include the following:

- Confirmation of how this development will meet the zero-carbon policy requirement in line with the Energy Hierarchy;

- Demonstrate the cooling hierarchy has been followed, and specify which overheating mitigation measures are proposed to reduce the overheating risk within the proposed design:

- A minimum site-wide carbon reduction of 7% under Be Lean with SAP 10.2 carbon factors, aiming for 15% reduction;

- GLA’s Carbon Emission Reporting Spreadsheet;

- Detailed BRUKL calculations for the non-residential element of the development for Be Lean, Be Clean and Be green scenarios;

- Location, specification and efficiency of the proposed ASHPs (Coefficient of Performance, Seasonal Coefficient of Performance, and the Seasonal Performance Factor), with plans showing the ASHP pipework and noise and visual mitigation measures;

- Specification and efficiency of the proposed Mechanical Ventilation and Heat Recovery (MVHR), with plans showing the rigid MVHR ducting and location of the unit;

- Details of the PV, demonstrating the roof area has been maximised, with the following details: a roof plan; the number, angle, orientation, type, and efficiency level of the PVs; how overheating of the panels will be minimised; their peak output (kWp)

- Specification of any additional equipment installed to reduce carbon emissions.

(b) The solar PV arrays and air source heat pumps must be installed and brought into use prior to first occupation of the relevant block. Six months following the first occupation of that block, evidence that the solar PV arrays have been installed correctly and are operational shall be submitted to and approved by the Local Planning Authority, including photographs of the solar array, installer confirmation, an energy generation statement for the period that the solar PV array has been installed, and a Microgeneration Certification Scheme certificate.

Should the agreed target not be able to be achieved on site through energy measures as set out in the aforementioned strategy, then any shortfall should be offset at the cost of £2,850 per tonne of carbon plus a 10% management fee. Should an increased level of CO2 reduction be achieved, any carbon offset payment would be reduced by £2,850 per tonne.

The final agreed energy strategy shall be installed and operation prior to the first occupation of the development. The development shall be carried out strictly in accordance with the details so approved and shall be operated and maintained as such thereafter. The solar PV array shall be also installed with monitoring equipment prior to completion and shall be maintained at least annually thereafter.

Reason: To ensure the development reduces its impact on climate change by reducing carbon emissions on site in compliance with the Energy Hierarchy, and in line with London Plan (2021) Policy SI2, and Local Plan (2017) Policies SP4 and DM22.

Overheating

Prior to above ground construction, a statement shall be submitted to and approved by the Local Planning Authority to demonstrate how the overheating risk has been minimised and mitigated on site in line with the London Plan Cooling Hierarchy. Details and specification of the mitigation measures shall be provided in support of this statement.

Reason: In the interest of reducing the impacts of climate change and mitigation of overheating risk, in accordance with London Plan (2021) Policy SI4, and Local Plan (2017) Policies SP4 and DM21.

Sustainability Strategy

Prior to above ground commencement of development, details of the sustainability strategy shall be submitted to and approved by the Local Planning Authority. This shall include specifications, plans and sections that demonstrate sustainable design, layout, construction techniques and proposed measures to improve the sustainability of the scheme including but not limited to sustainable transport, health and wellbeing, reduction of material use and waste, water consumption, and flood risk, drainage improvements, and biodiversity enhancement.

The report shall include:

- Urban greening and biodiversity enhancement measures;

- Details on electric vehicles charging points, cycle parking facilities;

- A target percentage for responsibly sourced, low-impact materials used during construction;

- Justification for the demolition of the existing buildings in terms of its impact on the whole life carbon of the development and the circular economy principles;

- Details on how any demolition materials can be reused;

- Details on how surface water runoff will be reduced and overall sustainable drainage strategy;

- Climate Change mitigation measures to be considered for the external spaces and the impact of the increase in severity and frequency of weather events on the building structures.

Reason: To ensure the development provides the maximum provision towards increasing the level of sustainability in line with London Plan (2021) policies G6, SI7 and Haringey Local Plan Policy SP4, DM21, DM25, and DM29.

Biodiversity Measures

a) Prior to the commencement of development, details of ecological enhancement measures and ecological protection measures shall be submitted to and approved in writing by the Council. This shall detail the biodiversity net gain, plans showing the proposed location of ecological enhancement measures, a sensitive lighting scheme, justification for the location and type of enhancement measures by a qualified ecologist, and how the development will support and protect local wildlife and natural habitats.

(b) Prior to the occupation of development, photographic evidence and a post-development ecological field survey and impact assessment shall be submitted to and approved by the Local Planning Authority to demonstrate the delivery of the ecological enhancement and protection measures is in accordance with the approved measures and in accordance with CIEEM standards.

Development shall accord with the details as approved and retained for the lifetime of the development.

Reason: To ensure that the development provides the maximum provision towards the creation of habitats for biodiversity and the mitigation and adaptation of climate change. In accordance with London Plan (2021) Policies G1, G5, G6, SI1 and SI2 and Local Plan (2017) Policies SP4, SP5, SP11 and SP13.

Urban Greening Factor:

Prior to completion of the construction work, an Urban Greening Factor calculation should be submitted to and approved by the Local Planning Authority demonstrating a target factor of 0.3 has been met through greening measures.

Reason: To ensure that the development provides the maximum provision towards the urban greening of the local environment, creation of habitats for biodiversity and the mitigation and adaptation of climate change. In accordance with London Plan (2021) Policies G1, G5, G6, SI1 and SI2 and Local Plan (2017) Policies SP4, SP5, SP11 and SP13.

BREEAM:

(a) Prior to ground works, a design stage accreditation certificate must be submitted to the Local Planning Authority confirming that the development will achieve a BREEAM “Very Good” outcome (or equivalent), aiming for “Excellent”. This should be accompanied by a tracker demonstrating which credits are being targeted, and why other credits cannot be met on site.

The development shall then be constructed in strict accordance with the details so approved, shall achieve the agreed rating and shall be maintained as such thereafter for the lifetime of the development.

(b) Prior to occupation, a post-construction certificate issued by the Building Research Establishment must be submitted to the local authority for approval, confirming this standard has been achieved.

In the event that the development fails to achieve the agreed rating for the development, a full schedule and costings of remedial works required to achieve this rating shall be submitted for our written approval with 2 months of the submission of the post construction certificate. Thereafter the schedule of remedial works must be implemented on site within 3 months of the Local Authority’s approval of the schedule, or the full costs and management fees given to the Council for offsite remedial actions.

Reason: In the interest of addressing climate change and securing sustainable development in accordance with London Plan (2021) Policies SI2, SI3 and SI4, and Local Plan (2017) Policies SP4 and DM21.