The application site lies within viewpoints 1, the strategically important view of Central London from Alexandra Palace and view 23, the locally important view of Alexandra Palace from the corner of Seven Sisters Road, Amhurst Park and Eade Road.

The site also lies near 13 designated heritage assets and 11 non-designated heritage assets, which are included on a table on page 13 of the THVIA. Alexandra Palace is not included in this list, even though the long-range views of the heritage asset are part of how its setting contributes towards its significance and is discussed below alongside the locally significant viewpoints. It is also noted that the locally listed No. 590 (former Weights and Measure Office) Seven Sisters Road would likely have intervisibility with the site.

Alexandra Palace & views

The view southwards from Alexandra Palace towards the City of London is a strategic view and is considered to be sensitive to change. The impact on this view is sufficiently demonstrated and discussed in Representative view 1 in Appendix C. The proposed development would be a visible addition to the skyline but would not mask any landmarks, obstruct part of the vista, and would be read as part of the general built form of the city. While there would be an impact on the view, this would be neutral.

In so far as it relates to this application, the architectural form and position of Alexandra Palace (a GII Listed Building) which allows long-range views of the listed building and contributes to the buildings landmark status. This contributes to the building’s significance, and many views of Alexandra Palace re considered to be locally important, including view 23 identified in policy DM5, which the development site lies within.

View 23 is taken form the corner of Seven Sisters Road, Amhurst Park and Eade Road towards Alexandra Palace. The THVIA states that this view is similar to representative 5, but it is not taken from the same point as that identified in DM9. However, given the sites location in relation to the viewpoint when viewed on site, it is unlikely to obstruct or impinge this view and would have a neutral impact.

Woodberry Down Community JMI School

The school is a GII post war school built as part of the London School Plan in the late 1940s as part of the large-scale estates under construction nearby and is the earliest surviving example. The aspects of the schools setting which contribute to its significance are located on the south side of the river including the estates the school was built for. The development site forms part of the wider industrial area to the north of the school. As per the assessment contained in the supplementary note on heritage effects does not contribute to the school’s significance and would be seen as part of a varied townscape and the development would have a neutral impact on its significance.

Stoke Newington Reservoirs, Filter Beds and New River Conservation Area

The Stoke Newington Reservoirs, Filter Beds and New River Conservation Area is unique and is of great historical importance, relating to the water supply of North London since the seventeenth century and contains historic infrastructure, several of which are listed buildings. The area directly adjacent to the site is the New River which was constructed in the C17 to supply water to north London and is part of the industrial heritage in this part of the city. The existing site does not contribute towards the significance of the conservation area and is seen as part of the wider industrial site. The proposed development would add additional mass and height along the northern boundary to the river, clearly demonstrated in representational view 6. This would be seen in conjunction with the existing taller buildings near Seven Sisters Road and would not harm the significance of the Conservation Area in itself. However, some care would need to be given to development of the wider area going forward to ensure that this does not cumulatively erode the open character of the north bank of the New River and views towards Alexandra Palace are not obscured.

Maynards Sweet Factory

Maynards Sweet Factory is a locally listed building located on the west of the development site and is part of the wider warehouse district. The historic similar land use and the low-level form of the area which allows an appreciation of the chimney as a prominent feature are important aspects of its setting. However, beyond these characteristics, the locally listed building is not sensitive to change to development within its setting. As the existing chimney would remain a prominent feature, the proposed development would have a neutral impact on the significance of the Maynards Sweet Factory

Woodberry Down Baptist Church

Woodberry Down Baptist Church is a locally listed building and is an attractive late C19 church designed by Paull and Bonella. The church was built in an imposing design and has some local landmark qualities. The church is prominent in several short to medium range views, including Vartry Road where the building terminates the view of the road westwards. Representative view 4 demonstrates the impact of the proposed development which given its scale on the skyline, would diminish the prominence and part of the landmark quality of the church. Accordingly, this would be considered to cause some harm to the significance of the non-designated heritage asset.

It is noted, contrary to the supplementary note on heritage effects  the development at the Woodberry Down Estate would not be seen in the view from Vartry Road.

No. 590 (former Weights and Measure Office) Seven Sisters Road & 100 Amhurst Park

Both 590 Seven Sisters Road and 100 Amhurst Park have intervisibility with the site, however it would be seen as part of the background townscape for both heritage assets. As per the assessment contained in the supplementary note on heritage effects does not contribute to either of the locally listed buildings’ significance and would be seen as part of a varied townscape and the development would have a neutral impact on its significance.

Summary

Overall the proposed development would have a neutral impact on the significance of the designated heritage assets, and the majority of the non-designated heritage assets, for which the site forms part of their setting. However it is considered that there would be some harm to the significance of Woodberry Down Baptist Church as it would  diminish the prominence and part of the landmark quality of the locally listed building and should be considered in line with paragraph 203 of the NPPF and policy DM9.