|  |  | Our ref: HGY/2024/0884 Contact: Adam Silverwood Date: 13/03/2025 |
| --- | --- | --- |
| Town and Country Act 1990 (As amended) |  |  |
| Location: | Council Depot, Ashley Road, Tottenham, London, N17 9DP |  |
| Proposal: | Partial approval of details pursuant to Condition 49(a) only (Living and Blue Roofs) attached to Planning Permission Ref: HGY/2022/0752 dated 31 August 2022. |  |
| Wording of condition 49(a): | 49) (a) Prior to the commencement of above ground works, details of the living and blue roofs must be submitted to and approved in writing by the Local Planning Authority. Living roofs must be planted with flowering species that provide amenity and biodiversity value at different times of year. Plants must be grown and sourced from the UK and all soils and compost used must be peat-free, to reduce the impact on climate change. The submission shall include: i) A roof plan identifying where the living and blue roofs will be located; ii) A section demonstrating settled substrate levels of no less than 120mm for extensive living roofs (varying depths of 120-180mm), and no less than 250mm for intensive living roofs (including planters on amenity roof terraces); iii) Roof plans annotating details of the substrate: showing at least two substrate types across the roof, annotating contours of the varying depths of substrate iv) Details of the proposed type of invertebrate habitat structures with a minimum of one feature per 30m2 of living roof: substrate mounds and 0.5m high sandy piles in areas with the greatest structural support to provide a variation in habitat; semi-buried log piles / flat stones for invertebrates with a minimum footprint of 1m2, rope coils, pebble mounds of water trays; v) Details on the range and seed spread of native species of (wild)flowers and herbs (minimum 10g/m2) and density of plug plants planted (minimum 20/m2 with roof ball of plugs 25m3) to benefit native wildlife, suitable for the amount of direct sunshine/shading of the different living roof spaces. The living roof will not rely on one species of plant life such as Sedum (which are not native); vi) Roof plans and sections showing the relationship between the living roof areas and photovoltaic array; and vii) Management and maintenance plan, including frequency of watering arrangements. viii) A section showing the build up of the blue roofs and confirmation of the water attenuation properties, and feasibility of collecting the rainwater and using this on site; Reason: To ensure that the development provides the maximum provision towards the creation of habitats for biodiversity and supports the water retention on site during rainfall. In accordance with London Plan (2021) Policies G1, G5, G6, SI1 and SI2 and Local Plan (2017) Policies SP4, SP5, SP11 and SP13. |  |

Carbon Management Response 05/2024

Documents submitted:

- 3742B-LB-XX-XX-D-A-453000_C1_Roof Types

- 3742B-LB-ZZ-05-D-A-720000_C2_Roof Details - SMJ Flat Roof

- 3742B-LB-ZZ-XX-D-A-711011_C2_Parapet Details - Block A1

- 3742B-LB-ZZ-ZZ-D-A-711010_C3_Parapet Details - Block A4

- 3742B-LB-ZZ-ZZ-D-A-711013_C2_Parapet Details - Over A4 Inset Balcony

- 3742B-LB-ZZ-ZZ-D-A-711014_C2_Parapet Details - A2 B3 Blue Roof

- ARD_PC 49a cover letter

- Location Plan

Response

Comments on plans and details for living and blue roofs:

- The applicant did not provide information in response to condition 49(a)i, iii, iv, v, vi, vii.

- The Roof Types drawing has shown a section of the biodiverse blue roof, however depth of the growing medium on this section was not provided. Please provide information in response to condition 49(a)ii and provide any other relevant roof type dimensioned sections (if there are more than one biodiverse roof types).

- The Roof Types drawing has included dimensioned sections of blue roofs, however there is no information on water attenuation properties, feasibility of collecting the rainwater and its use on site as requested in condition 49(a)viii.

Conclusion

Further information is required so we therefore do not recommend discharging Condition 49(a). The applicant should provide a response to the points raised above and issue updated plans/information where required.

Carbon Management Response 17/01/2025

Documents submitted:

- PR7976 ARD Blue Roof Calculation 100mm Storage Geocell.pdf

- PR7976 ARD Blue Roof Summary 100mm Storage REV0.pdf

- PR7976 Ashley Road Depot Specification Summary Rev2.pdf

- Zinco BPXA 100% Perennial Wildflower seed mix.pdf

Response

Applicant has provided specification summary identifying where living and blue roofs will be located.

The information has confirmed the storage geocell for blue roofs will be 100mm thick and the water attenuation capacity of all blue roofs.

The wildflower seed mix list has not provided all the information required in condition 49(a)v.

Actions:

- The applicant to provide information in response to condition 49(a) iii, iv, vi, vii.

- Applicant has not provided further information to confirm the depth of the growing medium. Please provide information in response to condition 49(a)ii and provide any other relevant roof type dimensioned sections (if there are more than one biodiverse roof types).

- The wildflower seed mix list only covers the information on seed. Applicant to confirm if any plug plant will be provided as per condition 49(a)v.

- The applicant has provided a response to the first half of condition 49(a)viii, but yet to confirm the feasibility of collecting the rainwater and its use on site. Please provide the information accordingly.

Conclusion

Further information is required so we therefore do not recommend discharging Condition 49(a). The applicant should provide a response to the points raised above and issue updated plans/information where required.

Carbon Management Response 13/03/2025

Documents submitted:

- 1113 Ashley Depot A3 Section.01 12Feb2025PR7976 ARD Blue Roof Summary 100mm Storage REV0.pdf (Biodiverse blue roof section with PV)

- 3742B-LB-BA-ZZ-D-A-411005_C1_Roof_Access_Strategy_-_Block_A.pdf

- 3742B-LB-ZZ-ZZ-D-A-411016_C1_Roof_Access_Strategy_-_Blocks_B_AND_C.pdf

- 3742B-LB-C1-05-D-A-120210_C2_Block C1 - Roof Plan.pdf (showing Biodiverse habitat substrate with sand pile)

- 3742B-LB-XX-XX-D-A-453000_C5_Roof Types.pdf

- 3742B-LB-CC-01-D-L-210013.pdf (Block C Podium Planting Plan)

- 3742B-LB-XX-XX-D-L-210003.pdf (Landscape Planting Plan)

- 3742B-LB-XX-XX-D-L-210004.pdf (Landscape Planting Scheme)

- Approved Ecological Enhancements by Ecologist.pdf (showing wildlife boxes)

- Approved Landscape Management Plan by Ecologist.pdf

- Plug Plant Specification from Zinco.pdf

- Radmat Blue Roof Maintenance Guidelines.pdf

- Radmat_Maintenance Guidelines_PQ6100.pdf

- 1113 Ashley Depot A3 Section.02 26Feb2025.pdf (showing varying substrate depth)

- 3742B-LB-XX-XX-D-L-200320.pdf (section of PV on roof with dimensions)

Response

| ii) | The submitted roof sections have demonstrated that sufficient substrate depths have been allowed. |
| --- | --- |
| iii) | Roof plan has shown two substrate levels. |
| iv) | Habitats and ecological enhancements installed as per the approved ecological enhancements in condition 50a. Further mounds added as per detail on architects layout for sandy piles. 1sqm of Rope coils and pebble mounds installed following spec of plug plants. |
| v) | Plug Plant density to be provided as per condition wording, attached in documents. |
| vi) | PV section included in the pack with dimensions. |
| vii) | Management and maintenance plans attached. |
| viii) | Unfeasible to use roof rainwater on site. Management and maintenance of roof water for SUDs only. The aspiration to use attenuated rainwater on site has not been captured in the proposed scheme. It is regrettably but the matter is now considered closed. |

Conclusion

The submitted information is acceptable and we can recommend that Condition 49(a) is discharged.